# VoIP and TV Troubleshooting Guide VPN

Description: User upgraded Rogers from XB6 to XB7 and VoIP and TV stopped working.

Causes:

- Modem has been changed.

Key Points:

- Rogers IP is Dynamic.

- Rogers Ethernet Port (Redline) must be connected to the Firewall WAN1.

- Firewall LAN1 must be connected to the SWITCH.

- Asus Router must be connected to the SWITCH.

- Home will NOT work if the IP is different from the Office.

- Firewall will not work unless Bridged Mode is ON from Rogers Modem.

- Rogers IP Information can be obtained through the Xfinity App.

Goal:

- Home and Office IP’s must be configured where both can communicate.

## IP References

### Rogers IP Information (Reference)

Subnet - 255.255.255.0

Router/Gateway - 10.0.0.1

DNS1 - <DNS_SERVER_1>

DNS2 - <DNS_SERVER_2>

DHCP Range - 10.0.0.2-253

WAN IPv4 – <PUBLIC_IP>

### Home IP Equipment Information

Unifi USG Pro 4
IP Address - 192.168.2.1

WAN1 – <PUBLIC_IP>

### Office IP Equipment Information

Unifi UXG Pro

IP Address - 192.168.1.1
WAN1 – <PUBLIC_IP>

WAN2 – <PUBLIC_IP>

USW 24 G1

IP Address - 192.168.2.1

## Troubleshooting Steps

- Ping 192.168.1.1 using CMD.

- Go to settings on laptop, Wi-Fi and connection, and manually setup DHCP.

- Try pinging 192.168.1.1 again.

If nothing else worked, do a factory reset on the Firewall.

- Change the old IP to the new one in Unifi.

- Send an adoption request to the office for approval after the factory reset from the Firewall.

- Check the Ethernet cables to see if it’s connected to the right ports on the Firewall, Switch, Asus Router and Rogers Modem.

- Wait for the Status to go online in Unifi.

- Check for adoption request in Unifi.

## Picture References